# Product Requirements Document (PRD)

## 1. Product Overview

### Product Name

SmartSpray — AI-Powered Precision Crop Spraying System

### Product Type

Flutter-based mobile application connected to a local AI/IoT system consisting of:

- Smartphone

- Laptop running AI/computer vision

- ESP32

- Servo motor

- Pump

- Spray nozzle

- Display

- Manual hardware controls

### One-line Product Definition

SmartSpray is a mobile control and monitoring application for an AI-powered precision spraying device that detects plant disease, estimates infection severity, recommends a spray level, and allows the farmer to automatically, semi-automatically, or manually control spraying.

# 2. Problem Statement

Traditional pesticide spraying generally treats an entire crop area uniformly.

This creates several problems:

- Healthy plants may receive unnecessary chemical treatment.

- Farmers have to visually inspect plants and make spraying decisions manually.

- Disease severity is difficult to estimate consistently.

- Excessive pesticide application can increase cost and environmental impact.

- There is limited digital tracking of what was detected and sprayed.

- A fully autonomous system may be too expensive or complex for small farmers.

The proposed system combines AI + computer vision + IoT + embedded control to make targeted spraying possible.

The mobile application acts as the farmer's interface to the system.

# 3. Product Vision

Create an affordable smart spraying system where a farmer can:

- Point the device at a plant.

- Let AI identify whether the plant is infected.

- See the disease and severity.

- Receive a spraying recommendation.

- Automatically spray, approve the spray, or control it manually.

- Monitor the device and review spraying history.

The long-term vision is a field-ready handheld/backpack precision spraying product.

Version 1 is a college/hackathon proof-of-concept, not a production agricultural product.

# 4. Target Users

## Primary User — Farmer / Field Operator

The primary user needs:

- Simple controls

- Minimal technical knowledge

- Fast disease information

- Clear spray recommendations

- Ability to override AI

- Reliable manual control

- Device status monitoring

The interface should therefore avoid technical AI terminology wherever possible.

Instead of:

Segmentation confidence = 0.934

show:

Disease detected — 93% confidence

## Secondary User — Agriculture Student / Researcher

For the prototype, students/researchers will also use the application to:

- Test the AI model

- Calibrate spray levels

- Monitor hardware

- Review detections

- Demonstrate the system

This user can have access to a more technical diagnostics section.

# 5. Product Goals

## Primary Goals

### G1 — AI-assisted spraying

Allow the user to use AI detection to determine whether spraying is required.

### G2 — Human control

Allow the farmer to override AI decisions.

### G3 — Device control

Allow the mobile app to control:

- Operating mode

- Servo position

- Pump

- Spray duration

### G4 — Real-time monitoring

Show:

- Device connection

- AI status

- Pump status

- Servo status

- Tank status, if available

- Battery status, if available

### G5 — Demonstrable end-to-end workflow

The complete system should successfully demonstrate:

Camera → AI → decision → phone → ESP32 → servo → pump → spray

# 6. Non-Goals

The first version is NOT intended to:

- Replace an agronomist.

- Prescribe legally approved pesticide dosages.

- Diagnose every crop disease.

- Work autonomously across an entire farm.

- Operate without human supervision.

- Guarantee crop treatment effectiveness.

- Automatically identify every plant species.

- Perform autonomous navigation.

- Replace commercial agricultural spraying equipment.

These limitations are deliberate.

# 7. System Architecture

The mobile app should not directly operate the pump.

Recommended architecture:

The laptop performs the computationally heavy AI work.

The ESP32 performs real-time hardware control.

The phone provides the user interface.

# 8. Operating Modes

This is a core product decision.

The application will support three modes.

## 8.1 AUTO Mode — Must Have

The AI makes the spraying decision.

Flow:

Camera

↓

AI

↓

Disease detected

↓

Severity estimated

↓

Spray recommendation

↓

ESP32

↓

Servo + Pump

Example:

Early Blight
Severity: 62%
Recommendation: Moderate spray

The system automatically executes the calibrated spray action.

# 8.2 ASSISTED Mode — Must Have

AI recommends the action, but the user confirms it.

Example:

Disease detected

Early Blight

Severity: 62%

Recommended:

MODERATE SPRAY

[ SPRAY ]    [ SKIP ]

This is arguably the safest and most practical mode for the prototype.

# 8.3 MANUAL Mode — Must Have

The user directly controls the hardware.

Controls:

- Servo left/right

- Pump ON/OFF

- Spray duration

- Stop

Example:

MANUAL MODE

Nozzle:

◀────────●────────▶

Duration:

500 ms

[ SPRAY ]

[ STOP ]

Manual mode is essential for:

- Hardware testing

- Calibration

- AI failure

- User override

- Demonstration

# 9. Core Features

## Feature 1 — Device Connection

### Priority: MUST HAVE

The application should identify whether the smart sprayer is connected.

Display:

Device: ONLINE 🟢

AI: READY 🟢

ESP32: CONNECTED 🟢

If disconnected:

Device: OFFLINE 🔴

[ RECONNECT ]

# 10. Dashboard

### Priority: MUST HAVE

The home screen should provide an immediate overview.

Example:

SMARTSPRAY

Device       🟢 Online

Mode         AUTO

AI           🟢 Ready

Plants scanned       42

Infected              9

Sprayed               7

Tank                  72%

Battery               84%

[ START AI SCAN ]

The dashboard should not overwhelm the user.

# 11. Live AI Detection

### Priority: MUST HAVE

The app should display the latest AI result.

Example:

PLANT DETECTED

Disease:

Early Blight

Confidence:

94%

Severity:

42%

Risk:

MODERATE

Recommendation:

MEDIUM SPRAY

If a camera stream can be safely exposed to the app later, a live camera view may be added.

For MVP, displaying the latest AI result is sufficient.

# 12. Disease Information

### Priority: MUST HAVE

Display:

- Disease name

- Confidence

- Infection severity

- Recommended action

Example:

Early Blight
Confidence: 94%
Estimated infection: 42%
Recommendation: Moderate spray

The app should explicitly label AI output as a model prediction, not a guaranteed diagnosis.

# 13. Spray Recommendation

### Priority: MUST HAVE

Convert AI severity into a prototype spray category.

Example:

0%              → NO SPRAY

1–25%           → LOW

26–60%          → MODERATE

>60%            → HIGH

These thresholds are prototype configuration values, not pesticide treatment standards.

The system should ultimately use crop/disease-specific agricultural recommendations before real-world pesticide deployment.

# 14. Spray Execution

### Priority: MUST HAVE

When spraying is authorized:

- Calculate calibrated spray level.

- Determine servo position.

- Move nozzle.

- Activate pump.

- Maintain pump for calibrated duration.

- Stop pump.

- Return status to application.

Example:

AI:

Severity = 42%

↓

SPRAY LEVEL:

MODERATE

↓

Servo: 87°

↓

Pump: ON

↓

Pump: OFF

↓

SPRAY COMPLETE

# 15. Manual Controls

### Priority: MUST HAVE

The application should provide:

- Pump ON

- Pump OFF

- Emergency software STOP

- Servo left

- Servo right

- Servo center

- Spray duration

The pump should never depend solely on the phone for safety.

A physical emergency stop/power cutoff should also exist on the hardware.

# 16. Emergency Stop

### Priority: MUST HAVE

There should be two layers.

### Software emergency stop

Mobile app:

[ EMERGENCY STOP ]

This sends an immediate stop command.

### Physical emergency stop

A physical switch disconnects pump power.

This protects against:

- Phone disconnect

- Wi-Fi failure

- Software crash

- ESP32 malfunction

- Pump stuck ON

# 17. Spray History

### Priority: MUST HAVE for MVP

Store:

- Timestamp

- Plant/detection ID

- Disease

- Severity

- Spray level

- Spray duration

- Spray status

Example:

18 Aug, 14:31

Early Blight

Severity: 42%

Moderate Spray

Completed

18 Aug, 14:34

Early Blight

Severity: 71%

High Spray

Completed

For the hackathon, SQLite is sufficient.

# 18. Statistics

### Priority: NICE TO HAVE

Show:

Today's activity

Plants scanned: 128

Healthy: 91

Infected: 37

Sprayed: 29

Estimated liquid used:

350 mL

This gives the judges a clear quantitative demonstration.

# 19. User Settings

### Priority: NICE TO HAVE

Possible settings:

- Spray calibration

- Servo calibration

- Disease model

- Severity thresholds

- Device name

- Connection settings

Example:

Spray Calibration

LOW       500 ms

MEDIUM   1000 ms

HIGH     1500 ms

These are prototype calibration settings, not pesticide dosage prescriptions.

# 20. Notifications

### Priority: NICE TO HAVE

Examples:

⚠️ Device disconnected

⚠️ Tank level low

⚠️ AI unavailable

✅ Spray completed

Not required for the first working prototype.

# 21. User Journey

## Step 1 — Launch app

User opens SmartSpray.

SmartSpray

Device: Connecting...

## Step 2 — Connect device

Application connects to the laptop/backend.

Device Connected 🟢

AI Ready 🟢

ESP32 Connected 🟢

## Step 3 — Select mode

User chooses:

AUTO

ASSISTED

MANUAL

Default should be ASSISTED during early testing.

## Step 4 — Start scanning

User presses:

START SCAN

The laptop camera begins detecting plants.

## Step 5 — AI detects plant

AI returns:

Plant detected

Disease: Early Blight

Severity: 47%

## Step 6 — Decision

### AUTO

System automatically sprays.

### ASSISTED

User sees:

Moderate spray recommended

[ SPRAY ] [ SKIP ]

### MANUAL

User controls the hardware.

## Step 7 — Physical action

ESP32:

- Positions servo.

- Activates pump.

- Sprays calibrated amount.

- Stops pump.

## Step 8 — Result

App displays:

SPRAY COMPLETE

Disease: Early Blight

Severity: 47%

Level: Moderate

Duration: 1.0 sec

## Step 9 — History

The event is stored.

User can review it later.

# 22. MVP Definition

The MVP should be much smaller than the complete vision.

### MVP must demonstrate exactly this:

Camera

↓

AI detects healthy/diseased plant

↓

Severity

↓

Flutter displays result

↓

User selects AUTO/ASSISTED/MANUAL

↓

FastAPI

↓

ESP32

↓

Servo

↓

Pump

↓

Spray

↓

Result logged

If this works reliably, the MVP is successful.

# 23. MVP Feature Priority

| Feature | MVP |
| --- | --- |
| Flutter Android app | ✅ Must |
| Device connection | ✅ Must |
| Dashboard | ✅ Must |
| AUTO mode | ✅ Must |
| ASSISTED mode | ✅ Must |
| MANUAL mode | ✅ Must |
| AI disease detection | ✅ Must |
| Severity estimation | ✅ Must |
| Spray recommendation | ✅ Must |
| Servo control | ✅ Must |
| Pump control | ✅ Must |
| Emergency stop | ✅ Must |
| Spray history | ✅ Must |
| Live video inside app | ❌ Not required |
| Cloud backend | ❌ Not required |
| AWS | ❌ Not required |
| GPS | ❌ Not required |
| Multiple crops | ❌ Not required |
| Multiple diseases | ❌ Not required initially |
| Soil sensors | ❌ Not required |
| Weather API | ❌ Not required |
| Farmer accounts | ❌ Not required |
| Push notifications | ❌ Not required |

# 24. Nice-to-Have Features

After the MVP works:

### Priority 1

- Live camera feed

- Better severity segmentation

- Multiple diseases

- Better spray calibration

- Real-time statistics

### Priority 2

- Soil moisture

- Temperature/humidity

- Chemical usage analytics

- Crop history

- Multiple user profiles

### Priority 3

- GPS

- Field mapping

- Cloud synchronization

- Remote monitoring

- Fleet/device management

# 25. Deliberately NOT Building in V1

This is extremely important for scope control.

## ❌ No AWS

There is no reason to put the AI in the cloud for the hackathon.

The laptop already provides sufficient compute.

## ❌ No autonomous field rover

The first product is handheld.

No:

- Autonomous driving

- GPS navigation

- Obstacle avoidance

- LiDAR

## ❌ No full agricultural prescription engine

The AI will not claim:

"Use exactly 17 mL of pesticide."

Instead:

"Prototype spray level: Moderate."

Real pesticide application requires crop-, disease-, product-, concentration-, nozzle-, and regulatory-specific validation.

## ❌ No 20-disease model

Start with:

one crop + one disease + healthy class

Then expand.

## ❌ No cloud-dependent control

The pump must continue to have a local control path.

Internet failure should not make the device uncontrollable.

## ❌ No complicated authentication

No farmer accounts, OTP, password recovery, etc. in V1.

## ❌ No payment/subscription system

Obviously outside the prototype scope.

# 26. Success Metrics

The product should be measured technically, not by how attractive the app looks.

## AI metrics

### Disease detection

Target:

≥90% precision on the controlled demonstration dataset

### Inference speed

Target:

≥10 FPS on the development laptop for the selected model/configuration, or sufficiently fast for the demonstration workflow.

### Severity estimation

For the prototype, compare predicted infected area with manually annotated test images.

Target:

reasonable correlation/error within a defined tolerance, rather than claiming clinical/agricultural accuracy.

# 27. Hardware Metrics

### Spray execution reliability

Target:

≥95% successful spray commands

Meaning:

Command sent → servo positions → pump activates → pump stops correctly.

### Servo positioning

Target:

repeatable positioning within approximately ±5°

for the prototype.

### Pump calibration

Measure actual liquid output.

Example:

500 ms → 4 mL

1000 ms → 8 mL

1500 ms → 12 mL

The actual values must be experimentally measured.

# 28. App Metrics

### Connection reliability

Target:

≥95% successful command delivery during controlled testing.

### Command latency

Target:

<500 ms for ordinary control commands on the local network.

### Crash-free demo

The application should complete an entire demonstration without crashing.

# 29. Hackathon Success Criteria

The strongest metric is whether a judge can understand the value in under one minute.

The demo should look like:

1. Point camera at healthy plant

↓

2. AI says HEALTHY

↓

3. No spray

4. Point at diseased plant

↓

5. AI says 62% infected

↓

6. Phone recommends MODERATE

↓

7. User presses SPRAY

↓

8. Servo moves

↓

9. Pump sprays

↓

10. Event appears in history

If you can execute that reliably, the product concept is clear.

# 30. Recommended Technology Stack

### Mobile

Flutter + Dart

### Backend

Python + FastAPI

### AI

Python + YOLO + OpenCV

### Severity

Segmentation / pixel-area analysis

### Hardware

ESP32 + Servo + MOSFET + Pump

### Communication

Local Wi-Fi + HTTP/WebSocket

USB Serial can be used initially because it is simpler.

### Database

SQLite

### AI Development

PyTorch

### Model annotation

CVAT or Roboflow

# 31. Product Architecture Principle

The most important architectural rule is:

The phone is the interface, not the brain and not the actuator.

The responsibilities are deliberately separated:

PHONE

User interface

↓

PYTHON

AI + decision engine

↓

ESP32

Real-time hardware control

↓

SERVO + PUMP

Physical action

This separation means you can replace any layer later without rebuilding the entire system.

# 32. Version Roadmap

## V0 — Engineering Prototype

- Laptop webcam

- YOLO

- ESP32

- Servo

- Pump

- Basic Python control

Goal: Make spray happen from an AI decision.

## V1 — Hackathon MVP

- Flutter app

- FastAPI

- AI

- AUTO / ASSISTED / MANUAL

- Servo/pump control

- History

- Emergency stop

- Polished UI

Goal: Demonstrate a complete intelligent product.

## V2 — Field Prototype

- Backpack-mounted system

- Better camera

- Battery

- Tank-level sensor

- Flow sensor

- Multiple nozzles

- Better disease model

- Better severity estimation

Goal: Demonstrate realistic field operation.

## V3 — Commercial/Product Research

- Crop-specific models

- Agronomic validation

- GPS

- Field mapping

- Cloud analytics

- Multiple devices

- Remote monitoring

- Regulatory/safety validation

- Commercial-grade hardware

Goal: Move from prototype toward an actual agricultural product.

# 33. Final Product Definition

### What are we building?

A Flutter mobile application that controls and monitors an AI-powered precision spraying device.

### Who is it for?

Farmers and field operators, with students/researchers as initial prototype users.

### What problem does it solve?

It reduces unnecessary blanket spraying by using AI to identify infected plants and determine targeted spraying actions.

### What makes it intelligent?

Computer vision + disease detection + severity estimation + automated actuator control.

### What makes it IoT?

The phone, laptop/backend and ESP32 communicate over a local network, allowing monitoring and control of the physical device.

### What makes it a complete product?

AI detection + decision making + physical spraying + mobile control + history + manual override + safety controls.

### What should the team focus on?

Don't build a huge app. Build the smallest system that makes this sentence true:

“I point the system at a plant, the AI understands its condition, the phone shows me the decision, and the physical device performs the appropriate targeted action.”

That is your MVP. Everything else is secondary.