Below is the engineering-grade Feature Ticket List I would use for SmartSpray. I’ve deliberately split it into vertical slices and dependencies rather than creating vague “build backend” tickets.

One correction to the PRD: do not build authentication, roles, WebSockets, history analytics, or polished Flutter screens before the hardware + AI vertical slice works. Those are secondary to proving the core product.

# SmartSpray — Complete Feature Ticket List

## Priority Legend

- 🔴 Must-have for launch — required for the hackathon MVP/demo

- 🟡 Should-have — valuable, but MVP can work without it

- 🟢 Nice-to-have — only build if the core system is stable

# Phase 0 — Project Foundation

## TICKET SS-001 — Monorepo Project Structure

Priority: 🔴 Must-have for launch
Dependencies: None

AI coding prompt:

Create the initial SmartSpray monorepo structure for a local AI + IoT application.

Create three independent components:

- backend/ — Python FastAPI application

- flutter_app/ — Flutter Android application

- esp32/ — PlatformIO ESP32 firmware

- ml/ — AI training/evaluation assets

The backend should use a clean layered structure separating API routes, services, models, schemas, AI, hardware communication, and configuration.

The ESP32 project should separate communication, servo, pump, and safety logic.

The Flutter application should use feature-based organization.

Add appropriate .gitignore files and a root README explaining how the three components communicate.

Do not implement business functionality yet.

Acceptance criteria:

- Repository builds without errors.

- Flutter project launches.

- FastAPI application starts.

- ESP32 PlatformIO project compiles.

- Directory structure matches the architecture.

- Secrets, model weights, SQLite database, and build artifacts are excluded from Git.

## TICKET SS-002 — Environment Configuration

Priority: 🔴 Must-have for launch
Dependencies: SS-001

AI coding prompt:

Implement centralized environment-based configuration for SmartSpray.

Backend configuration must support:

- host

- port

- database URL

- ESP32 serial port

- baud rate

- AI model path

- AI confidence threshold

- severity thresholds

- servo limits

- maximum pump duration

Provide .env.example.

Do not hardcode environment-specific values inside application code.

Acceptance criteria:

- Backend starts using environment configuration.

- .env.example contains every required variable.

- Missing critical configuration produces a clear startup error.

- Safety limits cannot silently fall back to unsafe values.

# Phase 1 — Hardware Foundation

## TICKET SS-003 — ESP32 Servo Controller

Priority: 🔴 Must-have for launch
Dependencies: SS-001

AI coding prompt:

Implement a reusable ESP32 servo controller.

Requirements:

- Initialize servo safely.

- Support configurable minimum and maximum angles.

- Reject angles outside the configured range.

- Provide a function to move the nozzle to a requested angle.

- Define a safe startup position.

- Avoid unnecessary jitter.

Keep servo logic independent from pump and serial communication.

Acceptance criteria:

- ESP32 boots with servo in safe position.

- Valid angles move the servo correctly.

- Invalid angles are rejected.

- Servo cannot be commanded beyond configured limits.

- Servo controller can be called independently from the pump.

## TICKET SS-004 — ESP32 Pump Controller

Priority: 🔴 Must-have for launch
Dependencies: SS-001

AI coding prompt:

Implement an ESP32 pump controller using the available relay/MOSFET hardware.

Provide:

- startPump()

- stopPump()

- isPumpRunning()

- configurable maximum runtime

The pump must default to OFF during boot.

The pump controller must never allow indefinite activation.

Acceptance criteria:

- Pump is OFF after ESP32 restart.

- Pump starts when explicitly commanded.

- Pump stops after requested duration.

- Pump cannot exceed the configured maximum runtime.

- Calling stop immediately disables the pump.

## TICKET SS-005 — ESP32 Hardware Safety Layer

Priority: 🔴 Must-have for launch
Dependencies: SS-003, SS-004

AI coding prompt:

Implement an independent ESP32 hardware safety layer.

It must enforce:

- maximum pump duration

- valid servo angle range

- pump OFF during emergency stop

- pump OFF after communication timeout

- pump OFF on boot

- rejection of malformed commands

Safety logic must execute independently from the FastAPI backend.

Acceptance criteria:

- A software failure cannot keep the pump running indefinitely.

- Pump automatically stops at the configured timeout.

- Invalid commands never activate the pump.

- Emergency stop prevents new spray commands.

- Restarting the ESP32 leaves the pump OFF.

# Phase 2 — ESP32 Communication

## TICKET SS-006 — ESP32 Serial Command Protocol

Priority: 🔴 Must-have for launch
Dependencies: SS-003, SS-004, SS-005

AI coding prompt:

Implement the SmartSpray USB serial command protocol.

Support commands such as:

SPRAY,<servo_angle>,<duration_ms>

STOP

STATUS

ESTOP

Implement acknowledgements:

ACK,SPRAY_STARTED

ACK,SPRAY_COMPLETED

ACK,STOPPED

ACK,ERROR,<code>

Validate every command before execution.

Reject unknown commands, malformed commands, invalid servo angles, excessive durations, and commands blocked by emergency stop.

Acceptance criteria:

- Commands can be sent over USB serial.

- Valid spray commands execute correctly.

- Invalid commands are rejected.

- ESP32 sends deterministic acknowledgement messages.

- Pump completion generates an acknowledgement.

- STOP immediately stops spraying.

## TICKET SS-007 — ESP32 Communication Timeout

Priority: 🔴 Must-have for launch
Dependencies: SS-006

AI coding prompt:

Add communication-loss safety to the ESP32.

Track the timestamp of the latest valid backend communication.

If the device is spraying and communication becomes unavailable beyond a configurable safety timeout, automatically stop the pump.

Do not stop the servo merely because communication is lost.

Make the timeout configurable.

Acceptance criteria:

- Communication timeout is detected.

- Pump stops automatically after timeout.

- Device returns to safe state.

- Timeout is reported through serial status.

- Normal communication does not trigger false stops.

## TICKET SS-008 — Physical Emergency Stop Integration

Priority: 🔴 Must-have for launch
Dependencies: SS-005

AI coding prompt:

Add support for a physical emergency-stop input that directly disables pump operation.

The emergency stop must take priority over all software commands.

When activated:

- pump immediately turns OFF

- new spray commands are rejected

- status reports emergency-stop state

Releasing the button must not automatically restart the pump.

Acceptance criteria:

- Physical emergency stop disables the pump.

- Backend cannot bypass the physical emergency stop.

- Pump remains OFF after emergency stop.

- A deliberate reset is required before spraying can resume.

# Phase 3 — Backend Foundation

## TICKET SS-009 — FastAPI Application

Priority: 🔴 Must-have for launch
Dependencies: SS-001, SS-002

AI coding prompt:

Build the initial SmartSpray FastAPI backend.

Requirements:

- /health

- API version prefix /api/v1

- centralized configuration

- structured logging

- exception handling

- request IDs

- CORS configuration for the Flutter development environment

Return consistent JSON responses.

Do not expose internal stack traces through API responses.

Acceptance criteria:

- FastAPI starts successfully.

- /health returns successful status.

- /api/v1 routes are registered.

- Unexpected errors return structured errors.

- Logs contain request information.

- Flutter development device can reach the API over local Wi-Fi.

## TICKET SS-010 — SQLite Database

Priority: 🔴 Must-have for launch
Dependencies: SS-009

AI coding prompt:

Add SQLite database support to the SmartSpray backend.

Create database models for:

- devices

- detections

- spray_events

- sessions

- calibrations

Add database initialization/migration support.

Use SQLAlchemy or an equivalent maintainable ORM.

Do not add PostgreSQL or cloud database dependencies.

Acceptance criteria:

- Database initializes automatically for development.

- Tables are created correctly.

- Backend can read/write all required entities.

- Database connection configuration comes from environment variables.

- Application restart does not lose existing records.

## TICKET SS-011 — Device Registry

Priority: 🔴 Must-have for launch
Dependencies: SS-010

AI coding prompt:

Implement the SmartSpray device registry.

Each device should have:

- ID

- name

- unique device UID

- online/offline status

- operating mode

- firmware version

- last seen timestamp

Add APIs:

GET /devices

GET /devices/{device_id}

GET /devices/{device_id}/status

For V1, support a single local device while keeping the data model extensible to multiple devices.

Acceptance criteria:

- Device can be registered.

- Device status can be queried.

- Last-seen timestamp is maintained.

- Offline state is represented correctly.

- Invalid device IDs return structured errors.

# Phase 4 — Backend ↔ ESP32

## TICKET SS-012 — Serial Manager

Priority: 🔴 Must-have for launch
Dependencies: SS-006, SS-009

AI coding prompt:

Implement a Python serial manager for communicating with the ESP32 over USB.

Requirements:

- configurable serial port

- configurable baud rate

- connection/disconnection handling

- command transmission

- acknowledgement reading

- timeout handling

- thread-safe or async-safe access

Do not expose raw serial operations directly to API routes.

Acceptance criteria:

- Backend can connect to ESP32.

- Backend detects ESP32 disconnects.

- Commands can be sent reliably.

- ACK responses are parsed.

- Serial errors are converted into application-level errors.

## TICKET SS-013 — Hardware Controller Service

Priority: 🔴 Must-have for launch
Dependencies: SS-012

AI coding prompt:

Build a hardware controller service that provides a clean backend abstraction over the ESP32.

Implement:

- spray(angle, duration)

- stop()

- emergency_stop()

- get_status()

The service must enforce backend safety limits before sending commands.

API routes must never communicate with serial hardware directly.

Acceptance criteria:

- Hardware operations work through the service.

- Invalid spray parameters are rejected.

- Hardware timeouts are handled.

- Duplicate concurrent spray operations are prevented.

- Hardware failures generate structured application errors.

# Phase 5 — AI Detection

## TICKET SS-014 — Webcam Capture Service

Priority: 🔴 Must-have for launch
Dependencies: SS-009

AI coding prompt:

Implement a Python/OpenCV webcam capture service for the laptop webcam.

Requirements:

- discover configured camera

- open camera

- capture frames

- detect unavailable camera

- release camera safely

- expose the latest frame to the AI inference pipeline

Do not stream the camera to Flutter in V1.

Acceptance criteria:

- Laptop webcam opens successfully.

- Frames can be captured continuously.

- Camera failure is reported clearly.

- Camera resources are released correctly.

- AI service can request frames.

## TICKET SS-015 — YOLO Model Loader

Priority: 🔴 Must-have for launch
Dependencies: SS-014

AI coding prompt:

Implement an AI model loader for a YOLO-based plant disease detection model.

Requirements:

- load model from configured path

- fail clearly if model is missing

- configurable confidence threshold

- model loaded once rather than per frame

- CPU inference support

- optional GPU support when available

Keep model loading separate from detection logic.

Acceptance criteria:

- Model loads successfully.

- Missing model produces a clear error.

- Model is not repeatedly reloaded.

- Confidence threshold is configurable.

- Inference runs on the laptop without cloud services.

## TICKET SS-016 — Plant Disease Detection

Priority: 🔴 Must-have for launch
Dependencies: SS-015

AI coding prompt:

Implement YOLO inference against webcam frames.

V1 should support the minimum disease scope defined by the trained model, preferably:

- Healthy

- Diseased

- optionally Early Blight

Return structured detection results containing:

- class

- confidence

- bounding box

- timestamp

Do not trigger spraying directly from the detector.

Acceptance criteria:

- Frames can be passed into inference.

- Detection results contain required fields.

- Confidence is returned.

- Bounding boxes are returned.

- Low-confidence detections are marked uncertain.

- Detector does not control hardware.

# Phase 6 — Severity & Decision Engine

## TICKET SS-017 — Infection Severity Calculation

Priority: 🔴 Must-have for launch
Dependencies: SS-016

AI coding prompt:

Implement infection severity estimation.

Calculate:

infection_percentage = diseased_area / total_leaf_area * 100

Use the available segmentation/mask information when supported by the model.

For the prototype use:

- 0% → NO SPRAY

- 1–25% → LOW

- 26–60% → MEDIUM

- 60% → HIGH

Treat these as prototype decision thresholds and clearly separate them from real agricultural pesticide dosage recommendations.

Return severity percentage and severity category.

Acceptance criteria:

- Severity percentage is calculated.

- Category is generated from configurable thresholds.

- Values are bounded between 0 and 100.

- Missing segmentation does not produce fabricated severity.

- Severity calculation is independently testable.

## TICKET SS-018 — AI Confidence & Uncertainty Handling

Priority: 🔴 Must-have for launch
Dependencies: SS-016, SS-017

AI coding prompt:

Implement AI uncertainty handling.

Automatic spraying must be blocked when:

- confidence is below configured threshold

- detection is inconsistent

- required plant/disease information is missing

- severity cannot be calculated reliably

Return an explicit UNCERTAIN decision state.

Acceptance criteria:

- Low-confidence detection never causes AUTO spray.

- Missing severity never causes AUTO spray.

- Uncertain results are visible to Flutter.

- Assisted mode can still present the recommendation to the operator.

## TICKET SS-019 — Detection Stability Filter

Priority: 🟡 Should-have
Dependencies: SS-018

AI coding prompt:

Implement temporal detection stability for webcam inference.

Require multiple consistent detections across consecutive frames before declaring a disease detection valid for automatic action.

Make:

- number of required frames

- confidence threshold

- allowed variance

configurable.

Prevent one-frame false positives from triggering automatic spraying.

Acceptance criteria:

- Single-frame false positives do not become stable detections.

- Consistent detections eventually become valid.

- Detection state resets when the plant changes.

- AUTO mode only acts on stable detections.

## TICKET SS-020 — Spray Decision Engine

Priority: 🔴 Must-have for launch
Dependencies: SS-017, SS-018

AI coding prompt:

Implement the SmartSpray decision engine.

Input:

- disease

- confidence

- severity

- operating mode

Output:

- spray decision

- spray level

- servo angle

- duration recommendation

- explanation

The engine must never claim to prescribe legally validated pesticide dosage.

Low confidence or invalid data must result in NO_AUTOMATIC_SPRAY.

AUTO mode can execute only when all safety conditions are satisfied.

ASSISTED mode should return a recommendation requiring operator approval.

MANUAL mode should bypass AI recommendation but remain subject to hardware safety limits.

Acceptance criteria:

- AUTO produces a decision only for valid detections.

- Low confidence prevents automatic spraying.

- ASSISTED produces recommendation without automatically spraying.

- MANUAL accepts explicit operator commands.

- Safety limits remain enforced in all modes.

# Phase 7 — Detection API

## TICKET SS-021 — Detection API

Priority: 🔴 Must-have for launch
Dependencies: SS-016, SS-017, SS-020

AI coding prompt:

Implement REST APIs for AI detection.

Add:

GET /detections

GET /detections/{id}

GET /detections/latest

The latest detection should expose:

- disease

- confidence

- severity

- spray recommendation

- timestamp

- uncertainty state

Persist valid detections to SQLite.

Acceptance criteria:

- Latest detection can be retrieved.

- Detection history can be queried.

- Detection records survive backend restart.

- API does not expose internal filesystem or model metadata unnecessarily.

# Phase 8 — Spray APIs

## TICKET SS-022 — Manual Spray API

Priority: 🔴 Must-have for launch
Dependencies: SS-013, SS-011

AI coding prompt:

Implement:

POST /api/v1/spray/manual

Request:

{

"device_id": "device-001",

"servo_angle": 90,

"duration_ms": 1000,

"command_id": "cmd-123"

}

Validate:

- device

- mode

- servo angle

- duration

- emergency stop state

- device availability

- duplicate command ID

- active spray state

Send the command through the hardware controller service.

Persist the spray event.

Acceptance criteria:

- Valid manual spray reaches ESP32.

- Invalid commands are rejected.

- Duplicate command IDs do not create duplicate sprays.

- Spray events are persisted.

- Hardware failures produce structured errors.

## TICKET SS-023 — Spray Stop API

Priority: 🔴 Must-have for launch
Dependencies: SS-013

AI coding prompt:

Implement:

POST /api/v1/spray/stop

The endpoint must immediately send a stop command to the ESP32.

It must be safe to call multiple times.

Update the current spray event appropriately.

Acceptance criteria:

- Pump stops immediately.

- Endpoint works during an active spray.

- Repeated STOP requests are safe.

- Hardware failure is reported.

- No confirmation mechanism delays the stop.

## TICKET SS-024 — Automatic Spray Execution

Priority: 🔴 Must-have for launch
Dependencies: SS-020, SS-022

AI coding prompt:

Implement automatic spray execution.

When AUTO mode receives a stable, valid disease detection:

- validate confidence

- calculate severity

- generate spray decision

- validate hardware limits

- send command to ESP32

- wait for acknowledgement

- record spray event

- publish completion state

Never execute automatic spray for uncertain detections.

Acceptance criteria:

- Valid AI detection can result in automatic spray.

- Low-confidence detection cannot.

- Duplicate detections cannot repeatedly trigger spraying.

- Pump stops after configured duration.

- Spray event is recorded.

# Phase 9 — Sessions & History

## TICKET SS-025 — Scan Sessions

Priority: 🔴 Must-have for launch
Dependencies: SS-010, SS-011

AI coding prompt:

Implement scan sessions.

Add APIs:

POST /sessions

GET /sessions

GET /sessions/{id}

POST /sessions/{id}/stop

A session should track:

- device

- mode

- start/end time

- plants scanned

- infected count

- sprayed count

- estimated liquid usage

- status

Detection and spray records should optionally reference the session.

Acceptance criteria:

- Session can be started.

- Detections can belong to a session.

- Spray events can belong to a session.

- Session can be stopped.

- Summary values are available.

## TICKET SS-026 — Spray History API

Priority: 🔴 Must-have for launch
Dependencies: SS-022, SS-025

AI coding prompt:

Implement:

GET /spray/history

Return spray events containing:

- timestamp

- disease

- severity

- spray level

- operating mode

- servo angle

- duration

- status

Support pagination or a reasonable limit.

Acceptance criteria:

- Completed sprays appear in history.

- Failed sprays are distinguishable.

- History survives restart.

- API returns UI-safe fields only.

# Phase 10 — Real-Time Events

## TICKET SS-027 — WebSocket Event System

Priority: 🟡 Should-have
Dependencies: SS-009, SS-021, SS-024

AI coding prompt:

Implement a WebSocket endpoint:

/ws/device/{device_id}

Publish events:

- DEVICE_CONNECTED

- DEVICE_DISCONNECTED

- PLANT_DETECTED

- DISEASE_DETECTED

- SEVERITY_UPDATED

- SPRAY_RECOMMENDED

- SPRAY_STARTED

- SPRAY_COMPLETED

- ERROR

Use a consistent event schema with event type, timestamp, device ID, and payload.

Acceptance criteria:

- Flutter/client can connect.

- Events are received without polling.

- Events contain valid JSON.

- Disconnects are handled gracefully.

- WebSocket failures do not affect hardware safety.

# Phase 11 — Flutter Foundation

## TICKET SS-028 — Flutter Application Shell

Priority: 🔴 Must-have for launch
Dependencies: SS-001

AI coding prompt:

Build the SmartSpray Flutter application shell.

Implement:

- Material theme

- Inter typography

- SmartSpray color palette

- responsive layout

- bottom navigation

- routing

Primary navigation:

- Home

- Detect

- Control

- History

Settings should be accessible from Home.

Use Riverpod for application state management.

Acceptance criteria:

- App launches on Android.

- Bottom navigation works.

- Theme matches SmartSpray design system.

- Routes are independent and maintainable.

- No hardcoded API URL exists in feature widgets.

## TICKET SS-029 — Flutter API Client

Priority: 🔴 Must-have for launch
Dependencies: SS-028, SS-009

AI coding prompt:

Implement a centralized Dio-based API client for SmartSpray.

Support:

- base URL from configuration

- request timeout

- JSON serialization

- API error parsing

- request logging in development

Create service methods for:

- devices

- detections

- spray

- sessions

- history

Keep network logic outside UI widgets.

Acceptance criteria:

- Flutter can communicate with FastAPI over local Wi-Fi.

- API errors become typed application errors.

- Timeout states are handled.

- API base URL is configurable.

# Phase 12 — Flutter Home

## TICKET SS-030 — Home Dashboard

Priority: 🔴 Must-have for launch
Dependencies: SS-029

AI coding prompt:

Build the SmartSpray Home dashboard.

Display:

- device status

- AI status

- ESP32 status

- pump status

- plants scanned today

- infected count

- sprayed count

- START SCAN

Use reusable status cards and clear loading/error states.

Do not create fake data in production mode.

Acceptance criteria:

- Dashboard displays real backend state.

- Offline device is visually distinguishable.

- Loading state exists.

- API error state exists.

- START SCAN initiates a session.

# Phase 13 — Flutter Detection

## TICKET SS-031 — Detection Result Screen

Priority: 🔴 Must-have for launch
Dependencies: SS-021, SS-029

AI coding prompt:

Build the Detect screen.

Display:

- plant detected state

- disease

- confidence

- severity percentage

- severity category

- spray recommendation

- AI uncertainty state

If the detection is uncertain, clearly communicate that automatic spraying is unavailable.

Acceptance criteria:

- Latest real detection appears.

- Confidence and severity are displayed.

- Uncertain detection has a distinct state.

- No automatic spray action is initiated by simply opening the screen.

## TICKET SS-032 — Assisted Spray Confirmation

Priority: 🔴 Must-have for launch
Dependencies: SS-022, SS-031

AI coding prompt:

Implement ASSISTED mode confirmation UI.

Example:

Early Blight

Severity: 62%

Confidence: 94%

Recommended: Moderate Spray

Buttons:

SPRAY

SKIP

SPRAY must call the backend spray endpoint.

SKIP must not activate hardware.

Display spray progress and completion state.

Acceptance criteria:

- User can approve recommendation.

- User can skip recommendation.

- Approval results in one spray command.

- Duplicate button taps do not duplicate sprays.

- Completion state is shown.

# Phase 14 — Flutter Control

## TICKET SS-033 — Operating Mode Selector

Priority: 🔴 Must-have for launch
Dependencies: SS-030

AI coding prompt:

Build the AUTO / ASSISTED / MANUAL mode selector.

Mode changes must be persisted through the backend.

Display the current active mode clearly.

Prevent unsafe mode transitions during active spraying.

Acceptance criteria:

- User can change modes.

- Backend is the source of truth.

- Current mode survives app refresh.

- Mode changes during active spraying are handled safely.

## TICKET SS-034 — Manual Hardware Controls

Priority: 🔴 Must-have for launch
Dependencies: SS-022, SS-023, SS-033

AI coding prompt:

Build MANUAL mode controls.

Provide:

- servo angle slider

- spray duration slider

- SPRAY button

- STOP button

Display configured min/max limits.

All commands must go through FastAPI.

Do not allow the Flutter app to directly communicate with ESP32.

Acceptance criteria:

- Servo angle can be selected.

- Spray duration can be selected.

- SPRAY sends validated command.

- STOP immediately stops the pump.

- UI prevents obviously invalid values.

- Backend remains authoritative.

## TICKET SS-035 — Emergency Stop UI

Priority: 🔴 Must-have for launch
Dependencies: SS-023, SS-034

AI coding prompt:

Add a prominent emergency stop control to the Control screen.

Emergency stop must:

- immediately call backend stop/emergency-stop functionality

- provide clear visual feedback

- require no confirmation dialog

- disable spray controls until the system is safely reset

The UI must make clear that the physical emergency stop remains the ultimate hardware safety mechanism.

Acceptance criteria:

- Emergency stop is always accessible during manual/active spray.

- No confirmation dialog delays activation.

- Pump stops.

- UI reflects emergency state.

- User cannot accidentally resume spraying without reset.

# Phase 15 — Flutter History

## TICKET SS-036 — Spray History Screen

Priority: 🔴 Must-have for launch
Dependencies: SS-026, SS-029

AI coding prompt:

Build the History screen.

Display spray events with:

- timestamp

- disease

- severity

- spray level

- mode

- duration

- status

Support loading, empty, error, and refresh states.

Acceptance criteria:

- Real backend history is displayed.

- Completed and failed events are distinguishable.

- Empty history has an appropriate empty state.

- Pull-to-refresh or equivalent refresh works.

# Phase 16 — Real-Time Flutter Integration

## TICKET SS-037 — Flutter WebSocket Client

Priority: 🟡 Should-have
Dependencies: SS-027, SS-029

AI coding prompt:

Implement a WebSocket service in Flutter using web_socket_channel.

Connect to:

/ws/device/{device_id}

Parse SmartSpray event messages.

Update Riverpod application state for:

- detection

- spray started

- spray completed

- device status

- errors

Handle reconnects without creating duplicate listeners.

Acceptance criteria:

- Flutter receives backend events.

- Detection screen updates without manual refresh.

- Spray status updates live.

- Reconnection works.

- WebSocket failure falls back gracefully to REST polling or refresh.

# Phase 17 — Device Pairing

## TICKET SS-038 — Local Device Pairing

Priority: 🟡 Should-have
Dependencies: SS-011, SS-028

AI coding prompt:

Implement local device pairing using a short-lived pairing code.

Flow:

- Backend generates pairing code.

- User enters code in Flutter.

- Backend validates code.

- Flutter receives a session/device token.

- Device becomes available to the app.

Pairing codes must expire and cannot be reused indefinitely.

Acceptance criteria:

- Valid pairing code links the phone to the device.

- Invalid code is rejected.

- Expired code is rejected.

- Pairing code cannot be reused after successful pairing.

- Token is stored securely on the device.

# Phase 18 — Security

## TICKET SS-039 — Backend Authorization Layer

Priority: 🟡 Should-have
Dependencies: SS-038

AI coding prompt:

Implement backend authorization for SmartSpray.

Define:

- ADMIN

- OPERATOR

- VIEWER

Enforce authorization server-side.

Operators may control spraying.

Viewers may only view dashboard, detections, history, and statistics.

Admins may configure devices and calibration.

Never trust role information supplied directly by Flutter.

Acceptance criteria:

- Unauthorized spray requests are rejected.

- Viewer cannot control hardware.

- Device access is validated server-side.

- Authorization happens before hardware commands.

## TICKET SS-040 — Secure Token Storage

Priority: 🟡 Should-have
Dependencies: SS-038

AI coding prompt:

Store the SmartSpray session token using Flutter secure storage.

Do not store tokens in plain SharedPreferences.

Handle:

- token missing

- token expired

- token revoked

- logout

Automatically clear invalid credentials when appropriate.

Acceptance criteria:

- Token is stored securely.

- Token is attached to authenticated API requests.

- Expired tokens are handled.

- Logout clears credentials.

# Phase 19 — Calibration

## TICKET SS-041 — Spray Calibration

Priority: 🟡 Should-have
Dependencies: SS-034, SS-010

AI coding prompt:

Implement basic spray calibration.

Store configurable parameters:

- servo minimum angle

- servo maximum angle

- default servo position

- pump duration limits

- optional estimated flow rate

Provide backend APIs and a simple admin UI.

Clearly label estimated liquid volume as an approximation unless physically calibrated.

Acceptance criteria:

- Calibration values can be stored.

- Hardware respects updated limits.

- Invalid calibration values are rejected.

- Calibration changes do not bypass safety limits.

# Phase 20 — Statistics

## TICKET SS-042 — Dashboard Statistics

Priority: 🟡 Should-have
Dependencies: SS-025, SS-026

AI coding prompt:

Implement:

GET /statistics/today

Return:

- plants scanned

- infected plants

- sprayed plants

- spray events

- estimated liquid usage

- infection rate

Use persisted database records as the source of truth.

Add corresponding Flutter dashboard cards.

Acceptance criteria:

- Statistics match underlying database records.

- Today's date boundaries are handled correctly.

- Zero-data days return valid zero values.

- Dashboard values update after new scans/sprays.

# Phase 21 — Reliability

## TICKET SS-043 — Idempotent Spray Commands

Priority: 🔴 Must-have for launch
Dependencies: SS-022

AI coding prompt:

Implement idempotency for all spray commands using command_id.

If the same command ID is received more than once, the backend must not execute the physical spray more than once.

Return the original command result for duplicate requests when possible.

Acceptance criteria:

- Duplicate HTTP requests do not produce duplicate sprays.

- Command IDs are persisted.

- Concurrent duplicate requests are handled safely.

- Client receives deterministic results.

## TICKET SS-044 — Stale Command Protection

Priority: 🔴 Must-have for launch
Dependencies: SS-022, SS-043

AI coding prompt:

Add timestamp/expiration validation to hardware commands.

Commands older than the configured validity window must be rejected.

When the ESP32 reconnects after being offline, old queued spray commands must never execute automatically.

Acceptance criteria:

- Expired commands are rejected.

- Offline/reconnect does not execute stale spray commands.

- Fresh commands continue to work.

- Rejection reason is exposed clearly.

## TICKET SS-045 — Backend Crash Recovery

Priority: 🔴 Must-have for launch
Dependencies: SS-013

AI coding prompt:

Implement backend startup and recovery logic that guarantees the hardware returns to a safe state.

On backend startup:

- connect to ESP32

- request hardware status

- ensure pump is OFF unless there is an explicitly valid active command

Do not replay unfinished spray commands automatically.

Acceptance criteria:

- Restarting FastAPI cannot cause unexpected spraying.

- Existing incomplete spray events are marked appropriately.

- ESP32 remains safe during backend restart.

- Backend can reconnect to the device.

# Phase 22 — Error Handling

## TICKET SS-046 — Unified Error Handling

Priority: 🔴 Must-have for launch
Dependencies: SS-009, SS-029

AI coding prompt:

Implement consistent SmartSpray error codes across backend and Flutter.

Include errors for:

- DEVICE_OFFLINE

- ESP32_TIMEOUT

- CAMERA_UNAVAILABLE

- AI_UNAVAILABLE

- LOW_CONFIDENCE

- INVALID_SERVO_ANGLE

- INVALID_DURATION

- SPRAY_ALREADY_RUNNING

- EMERGENCY_STOP_ACTIVE

- COMMAND_EXPIRED

- DUPLICATE_COMMAND

- UNAUTHORIZED

Flutter should display human-readable messages while preserving machine-readable error codes.

Acceptance criteria:

- Backend errors follow the documented schema.

- Flutter maps known error codes to useful messages.

- Raw stack traces are never shown to users.

- Hardware errors do not crash the Flutter application.

# Phase 23 — Demo Mode

## TICKET SS-047 — Developer Demo Mode

Priority: 🟡 Should-have
Dependencies: SS-021, SS-024, SS-030

AI coding prompt:

Implement a clearly identifiable developer/demo mode that can simulate:

- plant detection

- Early Blight

- confidence

- severity

- recommendation

- spray lifecycle

Demo mode must be disabled by default.

Clearly label simulated data as DEMO/SIMULATED.

Never present simulated detection as genuine AI output.

Acceptance criteria:

- Demo can run without camera/AI.

- Simulated data is clearly marked.

- Demo mode cannot silently activate in production.

- Demo spray can optionally use real hardware only after explicit configuration.

# Phase 24 — Testing

## TICKET SS-048 — ESP32 Firmware Tests

Priority: 🔴 Must-have for launch
Dependencies: SS-006, SS-007, SS-008

AI coding prompt:

Add firmware tests or testable logic for:

- invalid servo angles

- excessive pump duration

- STOP

- emergency stop

- communication timeout

- malformed commands

- unknown commands

- boot safety

Hardware-specific operations should be abstracted enough to test the safety logic independently.

Acceptance criteria:

- Safety-critical logic has automated tests.

- Maximum pump duration is tested.

- Invalid commands are tested.

- Emergency stop behavior is tested.

- Tests run consistently.

## TICKET SS-049 — Backend Unit Tests

Priority: 🔴 Must-have for launch
Dependencies: SS-020, SS-022, SS-043

AI coding prompt:

Create backend unit tests for:

- severity calculation

- decision engine

- confidence filtering

- authorization

- spray validation

- idempotency

- stale commands

- error handling

Hardware communication should be mocked.

Acceptance criteria:

- Critical decision logic has automated tests.

- Safety validation is tested.

- Duplicate commands are tested.

- Low confidence prevents automatic spraying.

- Tests do not require physical ESP32 hardware.

## TICKET SS-050 — End-to-End MVP Test

Priority: 🔴 Must-have for launch
Dependencies: SS-024, SS-032, SS-034, SS-036

AI coding prompt:

Create an end-to-end integration test plan for the complete SmartSpray MVP:

webcam/detection
→ severity
→ decision
→ FastAPI
→ ESP32
→ servo
→ pump
→ completion
→ database
→ Flutter display

Include AUTO, ASSISTED, and MANUAL flows.

Include failure scenarios:

- ESP32 disconnect

- camera failure

- low AI confidence

- emergency stop

- duplicate command

- backend restart.

Acceptance criteria:

- Complete happy path works.

- AUTO works.

- ASSISTED works.

- MANUAL works.

- Emergency stop works.

- Hardware disconnect fails safely.

- Spray history is generated.

# Phase 25 — UI Polish

## TICKET SS-051 — Production Demo UI Polish

Priority: 🟡 Should-have
Dependencies: SS-030, SS-031, SS-034, SS-036

AI coding prompt:

Polish the Flutter UI according to the SmartSpray design system.

Use:

- agricultural green primary palette

- white cards

- Inter typography

- 8-point spacing

- 12–16px card radius

- 48px touch targets

- clear success/warning/error states

Avoid neon cyberpunk styling and generic IoT dashboard aesthetics.

Focus on clarity during a live hackathon demonstration.

Acceptance criteria:

- Screens are visually consistent.

- Primary actions are obvious.

- Emergency stop is visually prominent.

- Loading/error/empty states are polished.

- UI works on common Android screen sizes.

# Phase 26 — Final Demo Flow

## TICKET SS-052 — Hackathon Demo Workflow

Priority: 🔴 Must-have for launch
Dependencies: All core MVP tickets

AI coding prompt:

Create a deterministic SmartSpray demonstration workflow.

Demonstration sequence:

- Start SmartSpray.

- Connect ESP32.

- Start scan session.

- Point webcam at a diseased plant.

- AI detects disease.

- Calculate severity.

- Flutter displays disease, confidence, severity, and recommendation.

- Demonstrate ASSISTED approval.

- FastAPI sends spray command.

- Servo moves.

- Pump activates.

- Pump automatically stops.

- Flutter displays SPRAY COMPLETE.

- History updates.

- Demonstrate emergency stop.

Add a documented pre-demo checklist and recovery procedure.

Acceptance criteria:

- Demo can be completed reliably.

- Operator knows exactly what action to take.

- Hardware failure has a recovery path.

- Emergency stop can be demonstrated safely.

- No cloud service is required.

# Final MVP Cut Line

If the team is short on time, stop after these tickets:

### 🔴 Absolutely Required

| Area | Tickets |
| --- | --- |
| Foundation | SS-001, SS-002 |
| Hardware | SS-003 → SS-008 |
| Backend | SS-009 → SS-013 |
| AI | SS-014 → SS-018 |
| Decision | SS-020 |
| Detection API | SS-021 |
| Spraying | SS-022 → SS-024 |
| Sessions/History | SS-025, SS-026 |
| Flutter | SS-028 → SS-036 |
| Reliability | SS-043 → SS-046 |
| Testing | SS-048 → SS-050 |
| Demo | SS-052 |

That is the actual MVP.

### 🟡 Build only after MVP works

- SS-019 — Detection stability

- SS-027 — WebSockets

- SS-038 — Device pairing

- SS-039 — Roles/authorization

- SS-040 — Secure tokens

- SS-041 — Calibration

- SS-042 — Statistics

- SS-047 — Demo mode

- SS-051 — UI polish

### 🟢 Explicitly defer

The PRD's optional features—live camera streaming, tank level, battery monitoring, GPS, weather integration, multiple crops, multiple diseases, cloud synchronization, fleet management, advanced analytics—should not enter the sprint until the physical end-to-end spray loop is reliable.

# Recommended Build Sequence

The tickets should be executed in roughly this order:

### The critical engineering milestone

Don't judge progress by “Flutter screen completed” or “YOLO model trained.”

The first real milestone is:

A real webcam detection produces a real decision that travels through FastAPI to the ESP32, physically moves the nozzle, runs the pump for a bounded duration, stops automatically, and records the event.

Once that works, everything else is mostly productization around a proven core.