# SmartSpray — Frontend Specification Document

Product: SmartSpray AI Precision Sprayer
 Platform: Flutter / Dart
 Primary device: Android phone
 Backend: FastAPI running on local laptop
 AI: Python + YOLO/OpenCV on laptop
 Hardware: ESP32 + servo + pump
 Connectivity: Local Wi-Fi + HTTP/WebSocket
 Design goal: Professional agricultural-tech product, not an “Arduino project” UI.

# 1. Frontend Product Definition

The SmartSpray app is the farmer/operator control center for the physical AI sprayer.

It should allow the user to:

- Connect to the sprayer.

- See device health.

- Start AI scanning.

- See disease/severity results.

- Use AUTO mode.

- Use ASSISTED mode.

- Use MANUAL mode.

- Stop spraying immediately.

- Review spraying history.

- View basic statistics.

- Configure permitted settings.

### Core interaction

The app does not directly communicate with the pump.

# 2. UX Principles

## P1 — Action first

The farmer should never have to navigate through multiple screens to stop spraying.

STOP must always be accessible during an active spray.

## P2 — AI should be understandable

Don’t show:

segmentation_confidence = 0.93472

Show:

Disease detected
 Confidence: 93%

## P3 — Don’t hide uncertainty

If AI is uncertain:

AI is uncertain
 Please review before spraying.

Not:

Disease detected

## P4 — Physical-world state is more important than app state

If the app says:

Pump OFF

but the ESP32 says:

Pump ON

the UI must show an error state, not blindly display the app’s requested state.

## P5 — One-handed operation

The operator may be holding the sprayer.

Important controls should be large and reachable.

# 3. Design Direction

### Visual style

Modern agricultural technology

Think:

- Clean

- Professional

- Natural

- High contrast

- Minimal

- Functional

- Slightly industrial

Avoid:

- Excessive gradients

- Neon “AI” aesthetics

- Futuristic dashboards

- Too many tiny charts

- Excessive animations

# 4. Color System

## Primary

### Smart Green

Primary: #2E7D32

Used for:

- Primary buttons

- Active states

- Brand elements

- Positive actions

### Primary Dark

#1B5E20

Used for:

- Pressed state

- Headers

- Strong emphasis

### Primary Light

#E8F5E9

Used for:

- Background highlights

- Healthy status

- Selected cards

# 5. Neutral Palette

Background:       #F7F9F7

Surface:          #FFFFFF

Surface Secondary:#F1F4F1

Text Primary:     #172018

Text Secondary:   #5F6B61

Text Disabled:    #9AA39C

Border:           #DDE3DE

Divider:          #E7EBE7

# 6. Status Colors

### Healthy / Success

#2E7D32

### Warning

#F9A825

### Moderate

#EF8F00

### Severe / Error

#C62828

### Info

#1565C0

### Offline

#757575

# 7. Dark Emergency Color

Emergency controls should be visually distinct.

Emergency Red: #D32F2F

Emergency Dark: #B71C1C

Do not use red for ordinary actions.

Red should mean:

Stop / danger / error

# 8. Typography

Use Inter.

Why:

- Highly readable

- Excellent for dashboards

- Good numerals

- Professional

- Works well on Android

- Strong hierarchy

### Typography scale

| Style | Size | Weight |
| --- | --- | --- |
| Display | 28 px | 700 |
| H1 | 24 px | 700 |
| H2 | 20 px | 700 |
| H3 | 18 px | 600 |
| Body Large | 16 px | 400 |
| Body | 14 px | 400 |
| Label | 13 px | 600 |
| Caption | 12 px | 400 |
| Numeric KPI | 28–32 px | 700 |

Avoid using more than 3–4 weights.

# 9. Spacing System

Use an 8-point spacing system.

4   → micro

8   → small

12  → compact

16  → standard

24  → section

32  → large

40  → major section

48  → screen-level

### Default screen padding

16 px

### Card padding

16 px

### Between major sections

24 px

# 10. Border Radius

Use consistent rounding.

Small controls: 8 px

Cards:          12 px

Large cards:    16 px

Bottom sheets:  24 px

Pills:          999 px

Don’t make every component extremely rounded.

# 11. Elevation

Keep shadows subtle.

### Standard card

Elevation: 1–2

### Modal

Elevation: 8+

### Floating action control

Elevation: 4

The app should look clean rather than “floating everywhere.”

# 12. Buttons

## Primary Button

Background: #2E7D32

Text: #FFFFFF

Height: 48 px

Radius: 12 px

Horizontal padding: 20 px

Example:

## Secondary Button

Background: transparent

Border: #2E7D32

Text: #2E7D32

## Destructive Button

Background: #D32F2F

Text: #FFFFFF

Use for:

STOP

Emergency Stop

Do not use for ordinary navigation.

## Icon Button

Minimum touch target:

48 × 48 px

Even if the visible icon is only 24 px.

# 13. Emergency Stop Component

This deserves special treatment.

During active spraying:

Properties:

- Red

- Large

- Always visible during spraying

- Confirmation should not be required

If someone needs to stop a physical pump, forcing them through:

“Are you sure?”

is bad UX.

Pressing Emergency Stop should immediately issue the stop command.

# 14. Inputs

## Text field

Height: 48–52 px

Radius: 10 px

Border: #DDE3DE

Focus:

Border: #2E7D32

Error:

Border: #C62828

# 15. Slider

Use for:

- Servo angle in manual mode

- Spray duration

- Calibration

Example:

Servo angle

20° ─────────●──────── 160°

87°

Always show the numerical value.

Never make the user guess where the slider is.

# 16. Cards

Cards are the main information containers.

### Device status card

Severity should have both:

- Percentage

- Human-readable category

# 18. Status Indicators

Use:

● ONLINE

● READY

● SPRAYING

● OFFLINE

● ERROR

Never rely on color alone.

For example:

🟢 ONLINE

rather than just a green dot.

This improves accessibility.

# 19. App Navigation

Use bottom navigation with 4 primary destinations:

Settings can be accessed from the Home screen rather than consuming a fifth primary navigation item.

### Navigation

Home

├── Device details

└── Settings

Detect

└── Detection details

Control

├── Auto

├── Assisted

└── Manual

History

└── Spray details

# 20. Home Screen

## Layout

# 21. Detection Screen

This is the most important screen for the AI story.

### Important

For the first version, the camera feed can remain on the laptop.

The app can show the AI result rather than streaming the entire video to the phone.

That reduces complexity considerably.

# 22. Control Screen

# Mode selector

# 23. Auto Mode

Show:

AI CONTROL ACTIVE

Camera:     READY

AI:         READY

ESP32:      CONNECTED

Current:

Plant #17

Disease:

Early Blight

Severity:

62%

Action:

MODERATE SPRAY

The app should not clutter the screen with manual controls while AUTO is active.

# 24. Assisted Mode

This is the ideal demo mode.

# 25. Manual Mode

# 26. History Screen

Use a timeline/list rather than a huge table.

TODAY

14:32

🔴 Early Blight

Severity: 62%

Moderate spray

Completed

14:29

🟢 Healthy

No spray

14:27

🔴 Early Blight

Severity: 41%

Moderate spray

Completed

Tap an event for details.

# 27. Statistics

Use only a few useful metrics.

Today's Summary

42 Plants scanned

9 Infected

7 Sprays

83% Healthy

A small chart can show:

Healthy     ███████████████

Infected    █████

Sprayed     ████

Do not build a complex analytics dashboard for V1.

# 28. Settings

Sections:

### Device

- Device name

- Connection

- Firmware version

### Spraying

- Spray calibration

- Maximum duration

- Servo limits

### AI

- Confidence threshold

- Disease model

- Severity thresholds

### About

- App version

- Model version

- Device version

Safety-critical settings should be Admin-only.

# 29. Modal Design

Use bottom sheets for short actions.

Example:

Don’t use modals for every tiny action.

# 30. Spray Confirmation

For ASSISTED mode, confirmation is explicit.

For AUTO, no confirmation should interrupt every plant.

For MANUAL, pressing SPRAY should execute immediately within safety limits.

For Emergency Stop, no confirmation.

# 31. Loading States

Never leave the user staring at a frozen button.

Examples:

Connecting...

◌

AI processing...

◌

Moving nozzle...

◌

Spraying...

◌

# 32. Empty States

History with no data:

No spray events yet.

Start scanning to see your

activity here.

Don’t display a blank screen.

# 33. Error States

Example:

# 34. UX Error Copy

Use plain language.

### Bad

ESP32_TIMEOUT

### Good

Device didn’t respond.
 Check the device connection.

### Bad

AI_CONFIDENCE < THRESHOLD

### Good

AI is uncertain.
 Review the plant before spraying.

### Bad

403 FORBIDDEN

### Good

You don’t have permission to control this device.

# 35. Accessibility

Minimum requirements:

- 48×48 touch targets

- High contrast

- Don’t communicate status using color alone

- Text labels for icons

- Large emergency stop

- Readable typography

- Avoid tiny dashboard text

# 36. Flutter Architecture

Use feature-based architecture.

lib/

│

├── main.dart

│

├── app/

│   ├── app.dart

│   ├── router.dart

│   └── theme.dart

│

├── core/

│   ├── constants/

│   ├── errors/

│   ├── network/

│   ├── storage/

│   └── utils/

│

├── models/

│   ├── device.dart

│   ├── detection.dart

│   ├── spray_event.dart

│   ├── session.dart

│   └── statistics.dart

│

├── services/

│   ├── api_service.dart

│   ├── websocket_service.dart

│   └── device_service.dart

│

├── features/

│   │

│   ├── dashboard/

│   │   ├── dashboard_screen.dart

│   │   ├── dashboard_provider.dart

│   │   └── widgets/

│   │

│   ├── detection/

│   │   ├── detection_screen.dart

│   │   ├── detection_provider.dart

│   │   └── widgets/

│   │

│   ├── control/

│   │   ├── control_screen.dart

│   │   ├── control_provider.dart

│   │   └── widgets/

│   │

│   ├── history/

│   │   ├── history_screen.dart

│   │   └── history_provider.dart

│   │

│   └── settings/

│       ├── settings_screen.dart

│       └── settings_provider.dart

│

└── widgets/

├── status_card.dart

├── detection_card.dart

├── mode_selector.dart

├── primary_button.dart

├── emergency_stop.dart

└── loading_indicator.dart

# 37. State Management

I recommend:

## Riverpod

Use it for:

- Device state

- Current mode

- Detection state

- WebSocket state

- Spray state

- User role

- History

Avoid putting API calls directly inside widgets.

Bad:

onPressed: () async {

// 100 lines of API logic

}

Better:

Widget

↓

Provider

↓

Service

↓

API

# 38. API Integration Architecture

The app communicates with one backend, not multiple third-party services.

Flutter

│

├── REST

│

└── WebSocket

│

▼

FastAPI

### Important conclusion

There are currently no third-party SaaS APIs required by the mobile app.

You previously considered AWS, but the architecture deliberately removed it because the AI runs locally on the laptop.

So:

- No AWS API

- No Firebase requirement

- No Google Maps

- No external AI API

- No cloud database

- No authentication provider

This is good for V1.

# 39. Internal API Base URL

Development:

http://<LAPTOP-IP>:8000/api/v1

Example:

http://192.168.1.10:8000/api/v1

WebSocket:

ws://<LAPTOP-IP>:8000/ws

Do not hardcode the IP.

Store it in configuration.

# 40. API Authentication

For MVP device pairing:

Authorization: Bearer <device_session_token>

Flutter stores the token using secure platform storage.

Do not store it in ordinary plaintext preferences.

# 41. API Response Format

Use a consistent structure.

### Success

{

"success": true,

"data": {

"device_id": "device-001",

"status": "online"

}

}

### Error

{

"success": false,

"error": {

"code": "DEVICE_OFFLINE",

"message": "The device is currently offline."

},

"request_id": "req-123"

}

# 42. Device Status API

### Request

GET /api/v1/devices/{deviceId}/status

### Response

{

"device_id": "device-001",

"status": "online",

"mode": "assisted",

"ai_status": "ready",

"pump_status": "off",

"servo_angle": 87,

"last_seen": "2026-08-18T18:50:00+05:30"

}

# 43. Latest Detection API

### Request

GET /api/v1/detections/latest

### Response

{

"id": "det-123",

"plant_id": 17,

"disease": "Early Blight",

"confidence": 0.94,

"severity_percent": 42,

"spray_level": "medium",

"timestamp": "2026-08-18T18:51:03+05:30"

}

# 44. Mode API

### Request

POST /api/v1/devices/{deviceId}/mode

Body:

{

"mode": "assisted"

}

Allowed:

auto

assisted

manual

Response:

{

"success": true,

"mode": "assisted"

}

# 45. Manual Spray API

### Request

POST /api/v1/spray/manual

Body:

{

"device_id": "device-001",

"servo_angle": 90,

"duration_ms": 1000,

"command_id": "cmd-123"

}

Backend validates everything before sending to ESP32.

Response:

{

"spray_id": "spray-001",

"status": "started"

}

# 46. Stop API

POST /api/v1/spray/stop

Response:

{

"status": "stopped"

}

This should be designed as an idempotent operation.

Calling STOP twice should still leave the pump OFF.

# 47. Emergency Stop API

POST /api/v1/device/emergency-stop

Response:

{

"status": "emergency_stop_active"

}

The UI should immediately reflect:

🔴 EMERGENCY STOP ACTIVE

# 48. History API

GET /api/v1/spray/history

Optional:

?limit=20

&offset=0

Response:

{

"items": [

{

"id": "spray-001",

"disease": "Early Blight",

"severity": 42,

"spray_level": "medium",

"duration_ms": 1000,

"status": "completed",

"timestamp": "2026-08-18T18:40:00+05:30"

}

],

"total": 1

}

# 49. WebSocket Integration

Connection:

ws://192.168.1.10:8000/ws/device/device-001

Events:

{

"event": "DISEASE_DETECTED",

"data": {

"disease": "Early Blight",

"confidence": 0.94,

"severity": 42

}

}

Then:

{

"event": "SPRAY_STARTED",

"data": {

"servo_angle": 87,

"duration_ms": 1000

}

}

Then:

{

"event": "SPRAY_COMPLETED",

"data": {

"spray_id": "spray-001"

}

}

Flutter should update its state from these events.

# 50. WebSocket Failure

If WebSocket disconnects:

App:

🟡 Reconnecting...

Use exponential backoff.

For example:

1 sec

2 sec

4 sec

8 sec

maximum 30 sec

Do not continuously reconnect every few milliseconds.

# 51. Third-Party Service Specification

For V1, the answer is intentionally simple:

| Service | Used? | Purpose |
| --- | --- | --- |
| AWS | ❌ | Not needed |
| Firebase | ❌ | Not needed |
| Google Maps | ❌ | Not needed |
| Google/Firebase Auth | ❌ | Not needed |
| OpenAI API | ❌ | Not needed |
| Cloud AI API | ❌ | Not needed |
| Cloud database | ❌ | Not needed |
| FastAPI | ✅ | Internal backend |
| ESP32 | ✅ | Internal hardware |
| YOLO | ✅ | Local AI |
| OpenCV | ✅ | Local computer vision |

FastAPI, YOLO and ESP32 are components of your own system, not third-party SaaS services.

# 52. Development Dependencies

These are libraries, not external services:

### Flutter

flutter_riverpod

dio

web_socket_channel

go_router

shared_preferences

flutter_secure_storage

intl

fl_chart

### Python

fastapi

uvicorn

pydantic

sqlalchemy

alembic

aiosqlite

pyserial

opencv-python

numpy

ultralytics

torch

The team should avoid adding packages unless there’s a clear reason.

# 53. Environment Configuration

Flutter should receive:

API_BASE_URL

WS_BASE_URL

Example:

API_BASE_URL=http://192.168.1.10:8000/api/v1

WS_BASE_URL=ws://192.168.1.10:8000/ws

Backend:

APP_HOST=0.0.0.0

APP_PORT=8000

DATABASE_URL=sqlite:///./data/smartspray.db

AI_MODEL_PATH=./data/models/best.pt

AI_CONFIDENCE_THRESHOLD=0.50

MAX_SERVO_ANGLE=160

MIN_SERVO_ANGLE=20

MAX_PUMP_DURATION_MS=3000

# 54. Responsive Design

The initial target is Android phones.

Design around:

360–430 px width

The app should still work on larger phones.

Don’t design exclusively around one specific phone.

Use Flutter’s responsive layout rather than fixed pixel positioning.

# 55. Portrait vs Landscape

### Default

Portrait

Better for handheld operation.

### Detection

Optional landscape mode if you eventually show a large live camera feed.

But don’t make landscape a V1 requirement.

# 56. Demo Mode

I strongly recommend adding a hidden/developer Demo Mode.

It should simulate:

Plant detected

↓

Disease detected

↓

Severity 62%

↓

Spray recommendation

↓

ESP32 command

Why?

If your camera suddenly fails during the hackathon, you can still demonstrate the UI.

But be honest:

“Demo mode is being used.”

Do not fake successful real-world spraying and present it as real.

# 57. Frontend Definition of Done

The Flutter frontend is ready for MVP when:

- App launches without errors

- Device connection is visible

- Dashboard works

- AUTO/ASSISTED/MANUAL switching works

- AI detection appears in the app

- Severity appears correctly

- Assisted SPRAY button works

- Manual servo control works

- Manual pump control works

- STOP works

- Emergency STOP is clearly visible during spraying

- History displays completed events

- Offline state is handled

- WebSocket reconnect works

- Unauthorized actions are rejected

- No API credentials are hardcoded

- UI doesn’t freeze while AI is processing

# 58. Final UI Architecture

The final application should feel like this:

And during a real operation:

📷 PLANT

↓

💻 AI DETECTION

↓

Disease: Blight

Severity: 62%

↓

📱 ASSISTED MODE

↓

[ SPRAY ] [ SKIP ]

↓

ESP32

↓

Servo → Nozzle

↓

Pump

↓

🌱 Spray

↓

✅ COMPLETED

↓

HISTORY

## Final recommendation

For V1, do not build a giant agricultural dashboard.

Build a small, extremely polished control application around the one impressive interaction:

AI detects a diseased plant → phone shows the result → farmer approves/automation triggers → nozzle moves → pump sprays → result is recorded.

That interaction should dominate the UI.

Everything else—statistics, settings, charts, elaborate profiles, cloud services—should stay secondary until that core loop is reliable.