# Technical Architecture Document

## 1. System Overview

### Product

SmartSpray — AI-Powered Precision Crop Spraying System

SmartSpray is a local-first AI + IoT system consisting of:

- A Flutter mobile application

- A Python/FastAPI backend

- A YOLO/OpenCV AI pipeline

- An ESP32 embedded controller

- A servo-controlled spray nozzle

- A DC pump

- A local database

- A laptop acting as the AI/backend edge computer

The system detects plant disease using a camera, estimates infection severity, determines a prototype spray level, and controls the physical sprayer.

The phone provides:

- Monitoring

- AUTO/ASSISTED/MANUAL mode selection

- Manual controls

- Detection information

- Device status

- Spray history

- Statistics

# 2. Architecture Decision

## Recommended architecture

* Authentication can be omitted from the first hackathon build.

### Core principle

The architecture deliberately separates responsibilities:

Flutter = User interface

FastAPI = Application/backend layer

AI engine = Computer vision + decision engine

ESP32 = Real-time hardware controller

Database = Persistent application state/history

Do not put AI logic inside Flutter.

Do not let Flutter directly control the pump.

Do not make AWS/cloud infrastructure a dependency for V1.

# 3. Recommended Technology Stack

## 3.1 Mobile Application

### Flutter + Dart

Recommendation: YES

Why:

- Single codebase

- Excellent Android support

- Fast UI development

- Easy API integration

- Good WebSocket support

- Good fit for a polished hackathon interface

- Can later target iOS/web/desktop if required

### Flutter packages

Recommended:

dio

go_router

flutter_riverpod

web_socket_channel

shared_preferences

json_annotation

freezed

freezed_annotation

intl

Optional:

fl_chart

permission_handler

connectivity_plus

Don't add packages just because they exist. Keep dependencies small.

# 4. Backend

## Python + FastAPI

### Why FastAPI?

Your AI is already Python-based.

Using FastAPI means:

Flutter

↓

FastAPI

↓

AI

instead of introducing another backend language.

Advantages:

- Python AI integration

- Async support

- REST API

- WebSocket support

- Automatic OpenAPI documentation

- Pydantic validation

- Easy local development

Recommended libraries:

fastapi

uvicorn

pydantic

pydantic-settings

sqlalchemy

alembic

aiosqlite

pyserial

opencv-python

numpy

ultralytics

python-multipart

# 5. AI Stack

## YOLO + OpenCV + PyTorch

### YOLO

Use an Ultralytics YOLO model for the initial prototype.

Potential pipeline:

Camera frame

↓

YOLO

↓

Plant / leaf detection

↓

Disease detection

↓

Segmentation

↓

Severity calculation

### OpenCV

Used for:

- Camera access

- Frame preprocessing

- Image resizing

- Drawing bounding boxes

- Displaying detection results

- Image transformations

- Camera calibration

### PyTorch

Used for:

- Model training

- Model loading

- Inference

- GPU acceleration

Your laptop GPU can be used during development.

# 6. AI Model Architecture

For V1, don't build a giant disease-classification system.

Start with:

Crop:

Tomato

Classes:

1. Healthy

2. Diseased

Then optionally identify:

Disease:

Early Blight

For severity:

Leaf area

↓

Diseased pixel area

↓

infection_percentage

Formula:

infection_percentage =

(diseased_area / total_leaf_area) × 100

Then map the percentage to a prototype spray level:

0%             → NONE

1–25%          → LOW

26–60%         → MEDIUM

>60%           → HIGH

These values are prototype rules, not real pesticide dosage recommendations.

# 7. Embedded System

## ESP32

ESP32 is responsible for deterministic physical control.

It should NOT run the YOLO model.

### ESP32 responsibilities

- Servo control

- Pump control

- Reading optional sensors

- Receiving commands

- Reporting device status

- Emergency stop handling

- Watchdog/failsafe

- Returning command acknowledgements

### Example command

SPRAY,87,1000

Meaning:

Servo angle = 87°

Pump duration = 1000 ms

The ESP32 should return:

ACK,SPRAY_STARTED

and later:

ACK,SPRAY_COMPLETED

# 8. Hardware Communication

## V1 — USB Serial

Recommended for the first working prototype.

Laptop

│

USB

│

ESP32

Why?

Because it is:

- Simple

- Reliable

- Easy to debug

- Low latency

- No Wi-Fi configuration required

## V2 — Wi-Fi

Later:

Laptop

│

Wi-Fi

│

ESP32

Use this when you want the physical device to be more independent.

For the hackathon, USB serial is the safer starting point.

# 9. Mobile ↔ Backend Communication

Use two communication methods.

## REST API

Use REST for:

- Getting device information

- Getting history

- Changing modes

- Starting/stopping operations

- Getting statistics

- Configuration

Example:

GET /api/v1/device/status

GET /api/v1/detections

GET /api/v1/sprays

POST /api/v1/device/mode

POST /api/v1/spray/manual

POST /api/v1/emergency-stop

## WebSocket

Use WebSocket for live events.

FastAPI

↕

WebSocket

↕

Flutter

Events:

DEVICE_CONNECTED

DEVICE_DISCONNECTED

PLANT_DETECTED

DISEASE_DETECTED

SEVERITY_UPDATED

SPRAY_RECOMMENDED

SPRAY_STARTED

SPRAY_COMPLETED

ERROR

This avoids repeatedly polling the backend.

# 10. Backend Internal Architecture

FastAPI should not contain everything in main.py.

Use layered architecture:

API layer

↓

Service layer

↓

Repository layer

↓

Database

Example:

Flutter

↓

API endpoint

↓

SprayService

↓

DeviceService

↓

ESP32Controller

↓

Hardware

This makes the system easier to maintain.

# 11. Complete Project Structure

smartspray/

│

├── README.md

├── .gitignore

├── .env.example

├── docker-compose.yml

├── Makefile

│

├── backend/

│   │

│   ├── requirements.txt

│   ├── pyproject.toml

│   │

│   ├── app/

│   │   ├── __init__.py

│   │   ├── main.py

│   │   │

│   │   ├── core/

│   │   │   ├── config.py

│   │   │   ├── logging.py

│   │   │   └── security.py

│   │   │

│   │   ├── api/

│   │   │   ├── router.py

│   │   │   │

│   │   │   └── v1/

│   │   │       ├── __init__.py

│   │   │       ├── device.py

│   │   │       ├── detection.py

│   │   │       ├── spraying.py

│   │   │       ├── history.py

│   │   │       ├── statistics.py

│   │   │       └── websocket.py

│   │   │

│   │   ├── models/

│   │   │   ├── device.py

│   │   │   ├── detection.py

│   │   │   ├── spray_event.py

│   │   │   ├── calibration.py

│   │   │   └── user.py

│   │   │

│   │   ├── schemas/

│   │   │   ├── device.py

│   │   │   ├── detection.py

│   │   │   ├── spraying.py

│   │   │   ├── calibration.py

│   │   │   └── user.py

│   │   │

│   │   ├── services/

│   │   │   ├── device_service.py

│   │   │   ├── detection_service.py

│   │   │   ├── spray_service.py

│   │   │   ├── calibration_service.py

│   │   │   └── statistics_service.py

│   │   │

│   │   ├── repositories/

│   │   │   ├── device_repository.py

│   │   │   ├── detection_repository.py

│   │   │   ├── spray_repository.py

│   │   │   └── calibration_repository.py

│   │   │

│   │   ├── ai/

│   │   │   ├── model_loader.py

│   │   │   ├── detector.py

│   │   │   ├── segmenter.py

│   │   │   ├── severity.py

│   │   │   └── decision_engine.py

│   │   │

│   │   ├── hardware/

│   │   │   ├── esp32_controller.py

│   │   │   ├── serial_manager.py

│   │   │   ├── servo_controller.py

│   │   │   └── pump_controller.py

│   │   │

│   │   └── websocket/

│   │       ├── manager.py

│   │       └── events.py

│   │

│   ├── migrations/

│   │   ├── alembic.ini

│   │   └── versions/

│   │

│   ├── tests/

│   │   ├── unit/

│   │   ├── integration/

│   │   └── api/

│   │

│   └── data/

│       ├── models/

│       ├── datasets/

│       └── captures/

│

├── flutter_app/

│   │

│   ├── pubspec.yaml

│   ├── analysis_options.yaml

│   │

│   ├── android/

│   ├── ios/

│   ├── web/

│   │

│   └── lib/

│       ├── main.dart

│       │

│       ├── app/

│       │   ├── app.dart

│       │   ├── router.dart

│       │   └── theme.dart

│       │

│       ├── core/

│       │   ├── constants/

│       │   ├── errors/

│       │   ├── network/

│       │   └── utils/

│       │

│       ├── models/

│       │   ├── device.dart

│       │   ├── detection.dart

│       │   ├── spray_event.dart

│       │   └── statistics.dart

│       │

│       ├── services/

│       │   ├── api_service.dart

│       │   ├── websocket_service.dart

│       │   └── device_service.dart

│       │

│       ├── features/

│       │   ├── dashboard/

│       │   ├── detection/

│       │   ├── control/

│       │   ├── history/

│       │   └── settings/

│       │

│       └── widgets/

│           ├── status_card.dart

│           ├── detection_card.dart

│           ├── spray_button.dart

│           └── mode_selector.dart

│

├── esp32/

│   │

│   ├── platformio.ini

│   └── src/

│       ├── main.cpp

│       ├── config.h

│       ├── communication.cpp

│       ├── communication.h

│       ├── servo.cpp

│       ├── servo.h

│       ├── pump.cpp

│       ├── pump.h

│       ├── safety.cpp

│       └── safety.h

│

├── ml/

│   │

│   ├── datasets/

│   ├── notebooks/

│   ├── training/

│   │   ├── train.py

│   │   ├── evaluate.py

│   │   └── export.py

│   └── configs/

│       └── dataset.yaml

│

└── docs/

├── architecture.md

├── api.md

├── hardware.md

├── ai.md

└── calibration.md

This may look large, but most folders will initially contain only a few files.

The point is to prevent the project from becoming one enormous unmaintainable script.

# 12. Database Architecture

For the hackathon:

### SQLite

Use SQLite initially because:

- Zero configuration

- Local

- Fast enough

- Easy backup

- No database server required

### Later

Move to PostgreSQL when the system becomes multi-user/cloud-based.

# 13. Database ER Diagram

users

│

│ 1:N

▼

devices

│

├───────────────┐

│               │

│ 1:N           │ 1:N

▼               ▼

detections     spray_events

│               │

│               │

└──────┬────────┘

│

▼

sessions

devices

│

│ 1:N

▼

calibrations

For the hackathon, users can be omitted entirely.

# 14. users Table

### Purpose

Stores people who use the system.

| Field | Type | Description |
| --- | --- | --- |
| id | UUID | Primary key |
| name | VARCHAR | User's name |
| email | VARCHAR | Login email |
| password_hash | VARCHAR | Hashed password |
| role | VARCHAR | farmer/admin/researcher |
| created_at | TIMESTAMP | Account creation |
| updated_at | TIMESTAMP | Last update |

### V1

You can skip authentication and this table.

Don't build login just to make the application look more professional.

# 15. devices Table

### Purpose

Represents each physical SmartSpray device.

| Field | Type | Description |
| --- | --- | --- |
| id | UUID | Primary key |
| name | VARCHAR | Device name |
| device_uid | VARCHAR | ESP32 unique identifier |
| status | VARCHAR | online/offline/error |
| mode | VARCHAR | auto/assisted/manual |
| firmware_version | VARCHAR | ESP32 firmware |
| last_seen_at | TIMESTAMP | Last communication |
| created_at | TIMESTAMP | Creation time |
| updated_at | TIMESTAMP | Last update |

Example:

id:

a1b2c3

name:

SmartSpray Prototype 01

device_uid:

ESP32-A82F91

status:

online

mode:

assisted

# 16. detections Table

### Purpose

Stores AI observations.

| Field | Type | Description |
| --- | --- | --- |
| id | UUID | Primary key |
| device_id | UUID | FK → devices |
| session_id | UUID | FK → sessions |
| disease | VARCHAR | Predicted disease |
| confidence | FLOAT | AI confidence |
| severity_percent | FLOAT | Estimated infection |
| spray_level | VARCHAR | none/low/medium/high |
| image_path | TEXT | Optional image |
| bbox_x | FLOAT | Bounding box |
| bbox_y | FLOAT | Bounding box |
| bbox_width | FLOAT | Bounding box |
| bbox_height | FLOAT | Bounding box |
| created_at | TIMESTAMP | Detection time |

Example:

Disease:

Early Blight

Confidence:

0.94

Severity:

42.5

Spray level:

medium

# 17. spray_events Table

### Purpose

Stores every physical spraying operation.

| Field | Type | Description |
| --- | --- | --- |
| id | UUID | Primary key |
| device_id | UUID | FK → devices |
| detection_id | UUID | Optional FK → detections |
| session_id | UUID | FK → sessions |
| mode | VARCHAR | auto/assisted/manual |
| servo_angle | FLOAT | Nozzle angle |
| duration_ms | INTEGER | Pump duration |
| estimated_volume_ml | FLOAT | Estimated liquid |
| status | VARCHAR | requested/started/completed/failed |
| error_message | TEXT | Failure details |
| started_at | TIMESTAMP | Start |
| completed_at | TIMESTAMP | Completion |

This table is critical because it records what the physical device actually did.

# 18. calibrations Table

### Purpose

Stores hardware calibration values.

| Field | Type | Description |
| --- | --- | --- |
| id | UUID | Primary key |
| device_id | UUID | FK → devices |
| calibration_type | VARCHAR | pump/servo/camera |
| parameter | VARCHAR | Specific parameter |
| value | FLOAT | Calibration value |
| unit | VARCHAR | ms/ml/degree/etc. |
| created_at | TIMESTAMP | Creation time |

Example:

pump

1000 ms

8.2 ml

Or:

servo

center

90°

# 19. sessions Table

### Purpose

Groups a continuous spraying/scanning operation.

Example:

A farmer scans one crop row for 10 minutes.

That entire operation becomes one session.

| Field | Type | Description |
| --- | --- | --- |
| id | UUID | Primary key |
| device_id | UUID | FK → devices |
| mode | VARCHAR | auto/assisted/manual |
| started_at | TIMESTAMP | Start |
| ended_at | TIMESTAMP | End |
| plants_scanned | INTEGER | Number scanned |
| infected_count | INTEGER | Number infected |
| sprayed_count | INTEGER | Number sprayed |
| estimated_liquid_ml | FLOAT | Total liquid |
| status | VARCHAR | active/completed/failed |

# 20. Relationships

### User → Device

One user can own multiple devices.

User 1 ───── N Devices

### Device → Detection

One device can generate many AI detections.

Device 1 ───── N Detections

### Detection → Spray Event

One detection may produce zero or one spray event.

Detection 1 ───── 0..1 Spray Event

For example:

Healthy:

Detection

↓

No spray

Diseased:

Detection

↓

Spray event

### Device → Calibration

One device can have multiple calibration records.

Device 1 ───── N Calibrations

### Session → Detection

One session can contain many detections.

Session 1 ───── N Detections

### Session → Spray Events

One session can contain many spray events.

Session 1 ───── N Spray Events

# 21. Database Indexes

At minimum:

devices.device_uid

devices.status

detections.device_id

detections.created_at

spray_events.device_id

spray_events.created_at

spray_events.session_id

sessions.device_id

sessions.started_at

This isn't critical for six plants in a hackathon, but it prevents bad database design later.

# 22. API Structure

Base URL:

/api/v1

## Device

GET    /devices

GET    /devices/{device_id}

GET    /devices/{device_id}/status

POST   /devices/{device_id}/mode

POST   /devices/{device_id}/connect

## Detection

GET    /detections

GET    /detections/{detection_id}

GET    /detections/latest

## Spray

POST   /spray/auto

POST   /spray/manual

POST   /spray/stop

GET    /spray/history

GET    /spray/{spray_id}

## Sessions

POST   /sessions

GET    /sessions

GET    /sessions/{session_id}

POST   /sessions/{session_id}/stop

## Statistics

GET    /statistics/today

GET    /statistics/session/{session_id}

## WebSocket

/ws/device/{device_id}

# 23. API Request Flow — AUTO

Camera

↓

AI detector

↓

DetectionService

↓

DecisionEngine

↓

SprayService

↓

ESP32Controller

↓

ESP32

↓

Servo

↓

Pump

The Flutter application receives the events through WebSocket.

# 24. API Request Flow — MANUAL

Flutter

↓

POST /spray/manual

↓

FastAPI

↓

Safety validation

↓

SprayService

↓

ESP32Controller

↓

ESP32

↓

Pump

The server should reject unsafe/invalid values.

For example:

servo_angle < 0

should be rejected.

Similarly:

duration_ms > configured_max_duration

should be rejected.

Do not allow the Flutter client to send arbitrary hardware commands without backend validation.

# 25. Safety Architecture

This is particularly important because the system controls a physical pump.

## Layer 1 — Flutter

User interface.

## Layer 2 — FastAPI

Validates:

- Mode

- Servo angle

- Pump duration

- Device state

## Layer 3 — ESP32

Validates again.

This is important because the ESP32 should never blindly trust a command.

Example:

MAX_SERVO_ANGLE = 160°

MAX_PUMP_DURATION = 3000 ms

The exact values should be determined by your hardware.

# 26. Emergency Stop

The emergency stop must work independently of the AI.

Recommended:

Battery

│

▼

Physical Emergency Stop

│

▼

Pump Power

Software:

Flutter

↓

STOP

↓

FastAPI

↓

ESP32

↓

Pump OFF

The physical emergency stop should remain effective even if:

- Laptop crashes

- Phone disconnects

- Wi-Fi fails

- FastAPI crashes

- ESP32 software hangs

# 27. Environment Variables

Create:

.env

Never commit it.

Use:

.env.example

in Git.

### Backend

APP_ENV=development

APP_HOST=0.0.0.0

APP_PORT=8000

DATABASE_URL=sqlite:///./data/smartspray.db

ESP32_CONNECTION=serial

ESP32_SERIAL_PORT=COM5

ESP32_BAUD_RATE=115200

AI_MODEL_PATH=./data/models/best.pt

AI_CONFIDENCE_THRESHOLD=0.50

SEVERITY_MILD_MAX=25

SEVERITY_MODERATE_MAX=60

MAX_SERVO_ANGLE=160

MIN_SERVO_ANGLE=20

MAX_PUMP_DURATION_MS=3000

CORS_ORIGINS=http://localhost:3000

On Linux:

ESP32_SERIAL_PORT=/dev/ttyUSB0

The COM port will vary by machine.

# 28. Flutter Configuration

Do not hardcode:

http://192.168.1.10:8000

inside random Dart files.

Use environment/configuration.

For development:

API_BASE_URL=http://192.168.1.10:8000/api/v1

WS_BASE_URL=ws://192.168.1.10:8000/ws

For production later:

API_BASE_URL=https://api.example.com/api/v1

WS_BASE_URL=wss://api.example.com/ws

# 29. Important Local Network Issue

Your phone and laptop must be able to communicate.

Recommended:

Wi-Fi

/         \

/           \

Phone         Laptop

│

ESP32

The laptop should listen on:

0.0.0.0

not only:

127.0.0.1

Otherwise the phone cannot reach the backend.

You may also need to allow port 8000 through the laptop's firewall.

# 30. CORS

FastAPI needs to allow the Flutter client if appropriate for your architecture.

Development:

CORS_ORIGINS=*

is convenient but should not be used in a production deployment.

For the hackathon, restrict it once the network setup is known.

# 31. AI Model Configuration

Don't hardcode model parameters.

Use configuration:

AI_MODEL_PATH=./data/models/best.pt

AI_CONFIDENCE_THRESHOLD=0.5

AI_IMAGE_SIZE=640

AI_DEVICE=0

For CPU:

AI_DEVICE=cpu

For GPU:

AI_DEVICE=0

The actual setting depends on your PyTorch/CUDA installation.

# 32. AI Data Storage

Don't store every camera frame in the database.

That's bad architecture.

Instead:

Database

↓

Detection metadata

and optionally:

data/captures/

detection_001.jpg

detection_002.jpg

The database stores:

image_path = data/captures/detection_001.jpg

Later, cloud object storage can replace local files.

# 33. Logging

Use structured logs.

Examples:

INFO  Device connected

INFO  AI model loaded

INFO  Plant detected

INFO  Disease detected

INFO  Spray command sent

INFO  Spray completed

WARNING ESP32 disconnected

ERROR Pump command failed

Never rely only on print() statements once the project becomes complicated.

# 34. Error Handling

The backend should define clear error codes.

Examples:

DEVICE_OFFLINE

AI_NOT_READY

INVALID_SERVO_ANGLE

PUMP_TIMEOUT

ESP32_TIMEOUT

SPRAY_ALREADY_RUNNING

EMERGENCY_STOP_ACTIVE

INVALID_MODE

Flutter can convert these into human-readable messages.

Example:

ESP32_TIMEOUT

becomes:

"Device did not respond. Check the ESP32 connection."

# 35. State Machine

Don't let the system randomly switch between states.

Use explicit states.

OFFLINE

↓

CONNECTING

↓

READY

↓

SCANNING

↓

DETECTION

↓

DECISION

↓

POSITIONING

↓

SPRAYING

↓

COMPLETED

↓

READY

Error:

ANY STATE

↓

ERROR

↓

SAFE_STOP

This will eliminate many integration bugs.

# 36. Mode State Machine

AUTO

ASSISTED

MANUAL

Switching modes should only happen when the pump is OFF.

For example:

SPRAYING

↓

User changes MANUAL

should be rejected.

First:

SPRAYING

↓

PUMP OFF

↓

READY

↓

MANUAL

# 37. Security

For a local hackathon system, keep security simple.

Minimum:

- Don't expose the backend to the public internet.

- Don't commit passwords/API keys.

- Validate all hardware commands.

- Restrict CORS.

- Don't expose arbitrary shell commands through APIs.

- Use a physical emergency stop.

You do not need a complicated OAuth/Auth0 system for V1.

# 38. Configuration Management

Have three conceptual environments:

development

testing

production

For the hackathon, you'll primarily use:

development

Keep configuration separate from application logic.

# 39. Testing Strategy

## AI tests

Test:

- Healthy plant

- Diseased plant

- Poor lighting

- Different angles

- Multiple leaves

- Partial occlusion

## Backend tests

Test:

- Device status

- Mode switching

- Spray command validation

- Emergency stop

- Database writes

- WebSocket events

## ESP32 tests

Test:

- Servo movement

- Pump ON/OFF

- Invalid commands

- Timeout

- Emergency stop

- Serial communication

## Integration test

The most important test:

Camera

↓

AI

↓

FastAPI

↓

ESP32

↓

Servo

↓

Pump

↓

Database

↓

Flutter

Run this repeatedly before the hackathon.

# 40. Development Order

Do NOT let all six people start randomly building everything.

Use this sequence.

## Stage 1 — Hardware

Make this work:

ESP32 → Servo

ESP32 → Pump

## Stage 2 — AI

Make:

Webcam → YOLO → Disease

work independently.

## Stage 3 — Backend

Make:

POST /spray

control the ESP32.

## Stage 4 — Flutter

Build:

Dashboard

Mode selection

Manual controls

Detection result

using fake data initially.

## Stage 5 — Integration

Connect:

AI → Backend → ESP32 → Hardware

## Stage 6 — Real Flutter integration

Connect:

Flutter → Backend → real system

## Stage 7 — Polish

Only after everything works:

- UI animations

- Charts

- Better graphics

- History

- Statistics

- Demo mode

# 41. Team Responsibilities

For six people:

### Member 1 — AI

Dataset

YOLO

Training

Evaluation

Inference

### Member 2 — Computer Vision

OpenCV

Segmentation

Severity

Camera

Calibration

### Member 3 — ESP32

Servo

Pump

Serial/Wi-Fi

Safety

Firmware

### Member 4 — Backend

FastAPI

Database

REST API

WebSocket

Hardware integration

### Member 5 — Flutter

UI

Navigation

API integration

WebSocket

Manual controls

### Member 6 — Integration/Product

Hardware enclosure

System integration

Testing

Demo workflow

Documentation

Presentation

# 42. What NOT to Do Technically

Avoid these common mistakes:

### ❌ One giant Python file

Don't put:

AI

FastAPI

serial

database

business logic

into main.py.

### ❌ Flutter directly talking to ESP32

Use:

Flutter → FastAPI → ESP32

instead.

### ❌ Cloud-first architecture

Don't introduce AWS just because it sounds impressive.

Your laptop already performs AI inference.

### ❌ Database for every video frame

Store detection events, not an infinite stream of images.

### ❌ AI controlling the pump without safety validation

Every physical command needs limits.

### ❌ Building the mobile app before testing the backend

First make:

Python → ESP32 → Pump

work.

Then expose it to Flutter.

# 43. Future Production Architecture

# 44. Final Recommended Stack

| Layer | Technology | Reason |
| --- | --- | --- |
| Mobile | Flutter/Dart | Cross-platform, fast UI |
| API | FastAPI/Python | Direct AI integration |
| AI | YOLO + PyTorch | Object detection/segmentation |
| CV | OpenCV | Camera/image processing |
| ORM | SQLAlchemy | Clean database abstraction |
| Migration | Alembic | Database version control |
| DB V1 | SQLite | Zero-config local database |
| DB later | PostgreSQL | Production scalability |
| Real-time | WebSocket | Live device/detection events |
| Hardware | ESP32 | Low-cost embedded control |
| Servo | PWM | Nozzle positioning |
| Pump | MOSFET/relay | Liquid actuation |
| Communication V1 | USB Serial | Reliable and simple |
| Communication V2 | Wi-Fi | Wireless device |
| ML training | PyTorch/Ultralytics | Model development |
| Deployment | Local laptop | No cloud dependency |
| Cloud V1 | None | Unnecessary complexity |

# 45. Final Architecture Decision

### The critical design principle

Keep V1 local.

Your laptop is already powerful enough to run the AI. Your phone is already powerful enough to provide the UI. Your ESP32 is already capable of controlling the hardware.

Adding AWS, cloud inference, complex authentication, GPS, multiple crops, and autonomous navigation at this stage would make the project more fragile, not more impressive.

Build the vertical slice first:

AI detection → severity → Flutter result → FastAPI → ESP32 → servo/pump → spray → database record.

Once that works end-to-end, every additional feature becomes an incremental improvement instead of another unresolved dependency.